Part #              Name                                        Date

Supplier:  ______________________________

SCAR Number _______________________

NOTE:  All Sections shown in italics must be responded to with the initial response (i.e. within 24 hours).

| YES | NA | NO | DOCUMENTATION RESPONSE EVALUATION |
| --- | --- | --- | --- |
|  |  |  | ## SECTION 1 - HEADER |
|  |  |  | 1.1 SCAR response is provided on Kenona SCAR form, issue number is present. All attachments submitted with the SCAR response must have the Kenona SCAR number referenced on them. |
|  |  |  | 1.2 SCAR response is on the supplier’s standard CAR form, the Kenona issued SCAR number (i.e. Issue No.) must be referenced / shown in the header information and all attachments, and at a minimum the supplier form must include all sections / fields as shown on the Kenona SCAR form |
|  |  |  | ## NOTE: Supplier must respond with a “YES” to either Item 1.1 or Item 1.2 or SCAR will be rejected. |
|  |  |  | ## SECTION 2 – TEAM MEMBERS |
|  |  |  | 2.1 SCAR response identifies a champion leading the investigation, including the person’s name, job title and phone number. |
|  |  |  | - All team members name(s) and job title(s) listed. NOTE: Based on the defect description, verify that the supplier is utilizing a cross-functional team approach. |
|  |  |  | ## SECTION 3 – DEFECT DETAILS |
|  |  |  | - Response identifies Kenona documented “DEFECT DETAILS” statement is shown on the SCAR form. This statement represents the Kenona initiators educated interpretation of the problem. |
|  |  |  | # SECTION 4 – SHORT TERM CONTAINMENT |
|  |  |  | 4.1 Containment response issued by supplier and received by Kenona issuing plant within 24 hours. |
|  |  |  | 4.2 Containment activities / response to include, at a minimum: |
|  |  | REJ | a) Product at Kenona user plant(s) – may include product Kenona has already shipped or at the customer(s). List quantity contained / suspect quantity found. |
|  |  | REJ | b) Product in transit from the supplier to the Kenona user plant(s). List quantity contained / suspect quantity found. |
|  |  | REJ | c) Product in finished goods at the supplier, or stored at an off-site warehouse. List quantity contained / suspect quantity found. |
|  |  | REJ | - Product in process or being produced at the supplier. List quantity of product contained / suspect quantity found. |
|  |  | REJ | e) Product at, going to or returning from a sub-supplier. List quantity of product contained / suspect quantity found. |
|  |  |  | f) Family parts that could contain the same problem (produced on the same process, but a different color, grade, configuration, etc.). List by part number the quantity of product contained / suspect quantity found |
|  |  |  | NOTE: Additional minimum containment actions: |
|  |  |  | a) Certify all shipments until corrective action(s) have been implemented and verified to prevent the recurrence. |
|  |  |  | - Green “X” (or obvious identifier that is clearly recognizable) identify all bar code labels indicating that the product has been certified. |
|  |  |  | HAS THE SUPPLIER ADDRESSED THE FOLLOWING CONCERNS IN THEIR RESPONSE? |
|  |  |  | 4.3 Based on the severity and extent of the problem, it may require additional measures, as follows: |
|  |  |  | a) If 100% of all product reflects the concern, can an interim standard be implemented that does not compromise the fit, form, function or appearance of the end assembly? Comment: |
|  |  |  | b) Can product be sorted / reworked? Comment: |
|  |  |  | c) Can problem be immediately corrected to eliminate the issue for subsequent production? Comment: |
|  |  |  | d) Can a poke-yoke be implemented at the supplier / Kenona to prevent exposure to Kenona customer(s)? Comment: |
|  |  |  | ## SECTION 5 – PROBLEM STATEMENT |
|  |  |  | 5.1 The supplier may determine that a modified / different description supports their internal investigation. This section is supplied to allow for the re-defining of the “DEFECT DETAILS” input from Section 3 (above). Supplier inputs own problem description |
|  |  |  | SECTION 6 – ROOT CAUSE(S) |
|  |  |  | NOTE: Root cause must address all of the defect details / problem statement identified on the SCAR. |
|  |  |  | 6.1 Potential Root Cause(s) – Initial evaluation of the source point or system root cause that contributed to the failure must be included with the initial response. The initial analysis will identify numerous potential root causes through the application of the basic problem-solving tools (5 whys/Cause & Effect/Is – Is Not/ Etc.) |
|  |  |  | 6.2 Root Cause(s) – The result will be the identification of the true root cause(s). This will result in 3 core documented responses. |
|  |  | REJ | a) Supplier details a fact-based response to the “DEFECT DETAILS” and/or PROBLEM DESCRIPTION |
|  |  | REJ | - Point of failure process or design root cause 1B. Escape or soft root cause, how did issue escape inspection and get shipped? |
|  |  |  | 3) Quality system element root cause (i.e. Policy, Procedure, Work Inst. |
|  |  |  | 4) The documented ability to turn the problem off and on again |
|  |  |  | ## SECTION 7 – CORRECTIVE ACTION |
|  |  | REJ | 7.1 The correct response must have a documented corrective action plan for all identified actual root causes shown in the previous steps. NOTE: The corrective action(s) must eliminate the root cause(s). |
|  |  |  | 7.2 The corrective action must address the condition(s) that resulted in the failure, including: |
|  |  | REJ | a) Point of failure process or design root cause(s) and soft (escape) cause(s). |
|  |  |  | b) System element root cause(s) |
|  |  | REJ | ## 7.3 The corrective action(s) must identify date(s) when permanent actions are 100% implemented. |
|  |  |  | ## SECTION 8 – VERIFY EFFECTIVENESS |
|  |  | REJ | - The correct response must identify the documented form or method of verification, who is responsible, the due date and actual dates completed. |
|  |  |  | 8.2 The verification of effectiveness shall include data gathered to validate the problem resolution and corrective action(s). The data to be submitted should include proofs such as X-Bar and R Charts, Cpk analysis, P-Charts, Inspection Reports, etc. |
|  |  |  | 8.3 The data to be submitted shall include a significant time period to be agreed to by Kenona QA (dependent of frequency / duration of production runs), with evidence that the issue has not occurred again. |
|  |  |  | NOTE: Based on the successful verification, Kenona QA will advise supplier on discontinuation of containment and certification. |
|  |  |  | ## SECTION 9 – PREVENT RECURRENCE |
|  |  | REJ | 9.1 The correct response will identify the documents that are (will be) updated, the person(s) responsible, the due date(s) and the actual completion date(s). |
|  |  |  | 9.2 Response may include updates of the system documents to support the institutionalization of the corrective actions. Documents to be updated include, but not limited to: |
|  |  |  | - Potential Failure Mode and Effects Analysis (PFMEA) |
|  |  |  | - Control Plan |
|  |  |  | - Process Flow Diagram |
|  |  |  | - Work / Operator Instructions |
|  |  |  | - Quality System Procedures / Elements |
|  |  |  | 9.3 Supplier repeats this activity on common parts / processes to prevent similar failures from occurring. |